AT-5555 PC software manual

Ⅰ Installation:

Run AT-5555Setup 1.00 (the host software) and follow the instructions.

Ⅱ USB driver’s installation

After the step I, please find the toolbar from the desktop. And, click “START-ALL PROGRAM-AT-5555” successively, then you can find two kinds of USB drivers available. Choose the USB driver (for windows XP or windows vista) that fits your computer’s operation system. Then install the USB driver. And restart your computer after you finish the installation.

Ⅲ Connecting the radio

Please insert the matched USB programming cable into USB port in the computer. Then remove the radio’s bottom cover and put the other end of USB into the three-core position where the arrow pointed to. After that, please plug in and turn on the radio.

Ⅳ Running Program

Please find the toolbar from the desktop. And, click “START-ALL PROGRAM-AT-5555” successively. And operate the AT-5555.

Ⅴ Interface Instructions:

- COM port setting:

Select COMMUNICATION PORT from SETUP menu. After selecting the matched port for your USB, click OK and exit. COM port setting is necessary only when the software is used for the first time.

2.  Channel information

When setting these options, there will be a channel editing interface on the right side.

①CH NO

This column stands for the number of channels displayed.

②BAND

It is a pull-down menu. You can select one band and edit the corresponding channel. The band number from A-F is matched with the respective band buttons of the radio. (20 channels 25.615-30MHZ)

③Emergency channel( channel 9)

Can be adjusted by the “EMG” button.

Push the “EMG” button, the channel become adjustable.

Then, you can set an emergency channel respectively all bands from band A to band F.

The following settings from ④ to ⒀ can be done separately for each channel.

④Frequency

Used for setting receiving frequency (25.615—30.105MHZ).

You can increase or decrease the frequency（ by 10HZ or its multiple）

⑤TX　 Frequency

Used for setting transmitting frequency(25.615—30.105MHZ).

You can increase or decrease the frequency（ by 10HZ or its multiple）.

⑥ BUSY function

Choose ENABLE, the radio can not transmit dring receiving The transmission is allowed after the previous signal disappears. Choose DISABLE, there would be no such function.

(Channels from 1 to 40 except channel 9 are DISABLE. The channel nine is ENABLE)

⑦VOIC

This function is not installed in this radio. But it keeps space for radio fan to install this function. The VOIC usage is same as ECHO.

⑧ECHO

Choose ENABLE, the ECHO function can be used by switch the echo button. choose DISABLE to shut off the function. (All channels are ENABLE)

⑨HI－CUT

Choose ENABLE to start HI-CUT function. Choose DISABLE to shut off the function. (All ENABLE)

⑩＋10KHZ

Choose ENABLEto start ＋10KHZ function. Choose DISABLE to shut off the function.(ENABLE)

⑾NB/ANL

Choose ENABLE to start NB/ANL function. choose DISABLE to shut off NB/ANL function. (Enable)

⑿RB: Choose ENABLE to start RB function. choose DISABLE to shut off RB function.(Enable)

⒀SCAN

ADD and DEL in scan list. When ADD is chosen, it will scan current channel when starting scan function. If DEL is chosen, when starting scan function, it will not scan current channel. (All are ADD)

3.-Option feature

When setting this item, there would be an optional menu as bellows:

⑴ Time out timer(TOT)(S)　   Setup TOT Frequency(factory setting: 180s)

30-600S     step  30S

⑵ RB Frequency(HZ)　        Setup RB Frequency

0.3-3KHz    step  10Hz

⑶ RB Time(ms)　           Setup RB Time

50-1000ms  step  50ms

⑷ CW side tone(HZ)　       Setup CW side tone Frequency

0.3-3KHz    step  10Hz

⑸ Frequency  step　       Setup PUSH KEY step

10k 　1k  100HZ

⑹ LCD Display              Setup LCD

OFF  or  ON

⑺ Scan mode              Setting scan mode

Squelch scan  or   Time Scan

⑻ Scan Resume  Time(s)   Setting scan comeback time

1-60S    step  1S

⑼ LCD Display in TX   Setting transmission contents on LCD

TX　Frequency display  or  SWR  LCD display

Or  battery voltage display  or  TOT remain time display

⑽ SWR protect value

Setting SWR protection value, when starting SWR protection, if the

value is over this setting value under transmission conditions, it will prohibit transmitting.

1.2-20　　step  0.1

⑾ Battery voltage protect value 1-min(v)

Setting DC voltage value, when starting DC voltage protection, if  the value is lower than this setting 
value under transmission conditions, it will prohibit transmitting.

10.5-17V　　step  0.1V

⑿ Battery voltage protect value 1-MAX(v)

Setting DC voltage value, when starting DC voltage protection, if the value is over the setting value under transmission conditions, it will prohibit transmitting

10.5-17V　　step  0.1V

⒀ FM　sq close delay time(ms)

Setting FM mode, when signal disappears, speaker closed time is put off.

5-300ms    step  5ms

⒁ AM　sq close delay time(ms)

Setting AM mode, when signal disappears, speaker closed time is put off.

5-300ms    step  5ms

⒂ CW/USB/LSB　sq close delay time(ms)

Setting CW/USB/LSB mode, when signal disappears, speaker closed time is put off.

5-300ms    step  5ms

⒃ Single Tone(HZ)

Setup Single Tone Frequency

0.3-3KHz    step  10Hz

⒄ MODE Option

If the following item is not ticked， the radio will prohibit implementing corresponding mode.

(Factory settings: all are selected)

⒅ Single change data

If this option is selected, only the current channel’s RB function is open when you push the RB button. Otherwise, RB function of all channels will be open when push RB button.

(Factory settings: both TX/RX  are open)

⒆ Single change RX/TX　Frequency

When the option is selected, you can only change receiving frequency during receiving .

Or, only change transmitting frequency duing transmitting.

When the option is not selected, you can change both receiving and transmitting frequency.

⒇ SWR enabled

When it is selected, it will start the SWR test protection.

（21）DC enabled

When it is selected, it will start DC test protection.

4.  Local & information

When it is chosen, you can register customer information in the radio for record.

- Embedded & Message

When this option is selected, software will read some radio information, such as, date of manufacture, software version, maintenance record, series number, etc.

information is fixed when manufactured and cannot be changed, but some can be adjusted by our supported software. This software is AT-5555SetOSetup 1.0.EXE.

Ⅵ Reading, Writing data

1、Writing data

After finish the above steps, click the PROGRAM pull-down menu and select the WRIT TO RADIO, then there would be a dialogue box as bellows:

Click OK, it will display programming process as bellows:

When it is finished, click Ok and exit.

The writing process is completed now. Then, you can write the same data into a series of radios.

2、Reading data

The data can be read when radio is well connected with the computer. The computer will read the data and save, or write the data into another radio, or rewrite the data after modification.

Please select READ FROM RADIO from host interface PORGRAM down menu ,

it shows “read data from radio ,do you wish continue?

Click OK , it will display data process:

When it is 100% finished, click OK to exit.

3. Data storage

To save the settings, select the SAVE or SAVE AS in the pull-down menu of the FILE of the host interface. And then, save the data according to the instructions.

4、Open data

To get previous data you saved, follow the steps: click the OPEN menu in the pull-down menu of the FILE in the host interface. Find the data name you want according to the instructions. Click OPEN, then the data will be saved automatically in the software.

5. Click the NEW menu in the pull-down menu of the FILE in the host interface, you can start a new data.

6. Loading factory initial data

Select INITIAL SETTING from the the SETUP menu in the host interface. the software would automatically upload the initial data which is the same as the data after the reset operation.